ɣ=Gamma

θ=Theta

Φ= Phi

μ=Mu

Angle of climb formula: Sin ɣ=T-DW

Angle of descent formula: Sin ɣ=D-TW

Thrust in a climb: T=D+W.Sin ɣ

Thrust in a descent: T=D-W.Sin ɣ

Lift in a climb/ Descent : L=W.Cos ɣ

Drag in gliding Decent: D=W.Sin ɣ

Drag in Power-on Decent: D=T+(W.Sin ɣ)

Gradient climb: G%= TW- DLx100

Gradient to degrees : Tan-1G%100=Climb angle

Rule of thumb: 5%=3°

Prop Vx: Vmd x 0.76=Vmp

Load factor formula: LF= LiftWeight

Lift in Turn: L=WCos ɣ

Load factor in turn = 1Cos Φ

Load factor in climb = = Cos Φ

Turn Radius Formula:  r= v2 (MSec)g .  tanΦ

Circle circumference: 2.π.r

Meters to Nautical mile: 1852m = 1nm

Feet to Nautical mile: 6080ft = 1nm

Kts to Meters / sec = kts x 0.5144

Rate of turn TAS/Radius x60: TASRadiusx60

Angle of bank for rate 1 turn : TAS10 +7°

Distance = Time x speed

LSS=38.95°K

Mach No.=TASLSS

Mach Cone: μ= 1Mach

Runway slope:  threshold 1 height-threshold 2 heightshortest TORA x 3.28x100

ROC (Rate of climb) : G% (in still air) x TAS

AoA: speed vector + longitudinal

Path angle: speed vector + horizontal

Pitch angle: longitudinal + horizontal

Tapered wing Aspect ratio= Wingspan2Wing Area

Aspect ratio= SpanAverage Chord

Wing Area= Span x Average cord

Taper Ratio= Tip ChordRoot Chord

maxbank angle for a given speed=Cos-1(1Speed given2)

Spin Down Lift off: 9xPSI

Spin Up Touch down: 7.7xPSI

| Aeroplane Category | Limit Load Factor + (Positive) | Limit Load Factor - (Negative) | Ultimate Load Factor + (Positive) | Ultimate Load Factor – (Negative) |
| --- | --- | --- | --- | --- |
| Transport | 2.5 | -1.0 | 3.75 | -1.5 |
| Normal (commuter) | 3.8 | -1.52 | 5.7 | -2.28 |
| Utility | 4.4 | -1.76 | 6.6 | -2.64 |
| Aerobatic | 6.0 | -3.0 | 9.0 | -4.5 |

PERF Formulas:

JET Specific fuel consumption= fuel flowunit of thrust

Prop Specific fuel consumption= fuel flowunit of Power

Specific Air Range= TASFuel Flow mass

Specific Range= NMFuel Flow mass

Acronyms

Ground Effect

L +      Lift

E +     Effective AoA

I -       Induced AoA

D -     Down Wash angle

ID -    Induced Drag

Shock Waves Expansion

S -    Static temperature

S -    Static Pressure

D -   Density

L -    LSS

M +  Mach No.

T -   Total Pressure

Shock Waves Oblique

Solid State Drives Long Memory Term

S +    Static temperature

S +    Static Pressure

D +   Density

L +    LSS

M -  Mach No.

T -   Total Pressure

Positive Gust Loads

Many Aviators Wont Even See Any Wind

M   -   Mass

A    -   Altitude

W  -   Wing loading

E   +  Eas

S   +  Surface Area

A   + Aspect Ratio

W  + Wing Area